VigilNetra Investigation Engine (VIE)

Video-LLM Accident Investigation Report Generator

Product Requirements Document — Software Track

| Author | Rahhul A. |
| --- | --- |
| Scope | Software only — no hardware/RPi build steps |
| Status | Draft v1.0 |
| Date | August 2026 |
| Related project | VigilNetra black-box (hardware + telemetry repo) |

# 1. Executive Summary

VigilNetra today records the event — dual-camera footage plus IMU/OBD/GPS telemetry, captured on a Raspberry Pi and downloaded after an incident. The Investigation Engine is the missing second half: you feed it that same clip, and it gives back a structured investigation report — what happened, in what order, at what speeds, and how confident the system is in each claim.

This document covers the software-only build: the video-LLM pipeline that ingests a clip and produces the report. It does not cover the Raspberry Pi capture hardware, which is treated as already solved.

# 2. Problem Statement

Right now, reconstructing what happened in an accident from raw dashcam-style footage is manual: someone has to watch the clip, guess speeds, and write up a narrative by hand. That's slow, inconsistent, and hard to trust when the footage itself is shaky, low-res, or poorly lit — which is common with low-cost RPi camera setups.

## 2.1 Goals

- Ingest a single accident/event clip and produce a structured, readable investigation report.

- Handle real-world footage quality: choppy frame rates, low resolution, motion blur — via conditional super-resolution.

- Ground every factual claim (especially speed) in something measurable, not just the LLM's impression of the video.

- Cross-check vision-derived kinematics against VigilNetra sensor logs when available.

- Attach a confidence level to every claim so the report doesn't overstate certainty.

## 2.2 Non-Goals (for this PRD)

- Hardware/firmware work on the Raspberry Pi capture unit — already built, out of scope here.

- Legal or forensic certification. The output is an AI-assisted draft report, not a certified accident-reconstruction document — this should be stated on the report itself.

- Fault/liability determination. The system describes what the footage shows; it does not assign blame.

- Real-time / live in-vehicle inference — this is a post-event, offline pipeline for now.

# 3. Primary User & Use Case

Primary user: You — reviewing your own or a client/demo clip after an event, or a fleet/insurance reviewer doing the same at larger scale later

Core flow: Upload clip → pipeline processes it → structured PDF investigation report is available for download

Secondary use: Interview/portfolio demo — a self-contained example of building a grounded, multi-stage video-LLM system, distinct from a generic "chat with your video" demo

# 4. System Overview

The pipeline is a single compiled LangGraph StateGraph — matching the pattern you're already standardizing on in the AI Test Engineering Assistant project — with nine sequential/branching nodes:

- Ingest → validate clip + sensor log (IMU/GPS/OBD), persist metadata

- Quality gate → fisheye undistortion, conditional super-resolution + stabilization

- Detect & track → per-object trajectories relative to the ego frame

- Ego-motion → ego speed/heading series from OBD + GPS + IMU — the primary kinematic input, not a cross-check

- Kinematics fusion → relative vision motion + ego-motion → absolute speed with propagated error bounds

- Video-LLM reasoning → grounded, timestamp-cited event narrative (front camera) plus driver-state observations (driver-facing camera)

- Claim verification → ground every claim, attach confidence bands; reject absolute speeds with no ego-motion provenance

- Report generation → structured PDF

- Persist & serve → dashboard/API delivery

Every node emits to a single unified LangSmith trace — no manual create_run calls layered on top, the same fix you're already making elsewhere.

# 5. Recommended Technology Stack

| Component | Recommended | Alternative | Why |
| --- | --- | --- | --- |
| Video-LLM core (event reasoning) | Qwen3-VL 32B/72B, self-hosted | Gemini 2.5/3 Pro API | Qwen3-VL has native second-level timestamp grounding and up to 1M-token context, so it can reference exact moments in the clip instead of vague descriptions; self-hosting keeps footage off third-party servers, which matters for incident evidence. Gemini is a strong fallback when you need higher zero-shot accuracy and don't mind an API call. |
| Super-resolution (choppy/low-res input) | FlashVSR (diffusion-based, real-time) | Real-ESRGAN (GAN-based, lighter) | FlashVSR is a 2026 one-step diffusion VSR model built for streaming/real-time upscaling with temporal consistency, which matters for video (not just single frames). Real-ESRGAN is a solid fallback on weaker hardware — GAN-based, frame-by-frame, well-documented. |
| Object detection | YOLOv11 | RT-DETR | Fast, well-supported, good vehicle/pedestrian accuracy at real-time speed. |
| Multi-object tracking | ByteTrack / BoT-SORT | DeepSORT | Keeps stable IDs across occlusion (common in crash footage where vehicles overlap). |
| Monocular depth / calibration | Depth Anything V2 + homography calibration | Manual camera calibration (if intrinsics known) | You already know this territory from the KITTI/Velodyne calibration work at CSIR-4PI — same geometry-before-ML principle applies here. Keyed per device_id in a DeviceProfile registry (mount height, pitch, yaw), not per clip. |
| Fisheye undistortion | OpenCV fisheye calibration (checkerboard, per-device) | - | Runs first, before every downstream vision step, non-negotiable — a homography fitted to a warped ground plane produces speeds that look plausible but are wrong. |
| Ego-motion extraction | Custom OBD/GPS/IMU parser (ego.py) | - | Ego speed is measured, not estimated, so the vision layer only has to solve the easier relative-motion problem; error on the final absolute speed is dominated by the vision term instead of two compounding estimates. |
| Motion cross-check | RAFT optical flow | Farneback (classical) | A second, independent velocity estimate to sanity-check the homography-based number before it goes in a report. |
| Orchestration | LangGraph (compiled StateGraph) | Manual pipeline / Celery chain | Matches the pipeline pattern you're already refactoring into for the AI Test Engineering Assistant — one compiled graph, one clean trace, no manual for-loop fallback. |
| Backend framework | FastAPI (Python 3.11) | - | Consistent with your existing stack (LeaRN, BashIn, AI Test Engineering Assistant). |
| Structured + relational data | Supabase / PostgreSQL | MongoDB (for raw perception blobs) | Structured incident/report metadata fits relational; large per-frame detection blobs can live in Mongo or object storage if they get heavy. |
| Object storage | Supabase Storage / S3-compatible | Local disk (dev only) | Raw clips, SR-upscaled clips, and generated PDFs need durable storage outside the app server. |
| Report rendering | Jinja2 templates → WeasyPrint (HTML→PDF) | ReportLab | Easiest way to get a clean, styled investigation-report PDF with tables and annotated evidence frames. |
| Tracing / eval | LangSmith | - | Same tool you're already using; one unified nested trace per investigation run. |
| Frontend | React + Vite dashboard | - | Matches your LeaRN/AI Test Engineering Assistant frontend stack. |
| Deployment | Docker Compose; cloud GPU burst (RunPod/Modal) for 72B inference | Fully local on RTX-class GPU with the 32B/quantized variant | Keep a local dev/demo path with the smaller Qwen3-VL variant so the whole thing runs on your desktop when you need to show it off. |

# 6. Core Design Principles

- Geometry before ML. Speed and distance claims come from calibrated computer vision (homography + optical flow), not from the LLM eyeballing the footage. The LLM narrates and reasons over numbers the pipeline already trusts — same principle you've been applying on the LiDAR/camera fusion side at CSIR-4PI.

- Ego-motion is ground truth, not a cross-check. VigilNetra logs OBD speed, GPS speed, and IMU acceleration alongside the video, so ego speed is measured rather than estimated. Absolute speed of another vehicle is ego_speed(OBD/GPS) ⊕ relative_speed(vision); when no sensor log exists, the pipeline still runs but every absolute speed is flagged method="vision-only", confidence="low".

- Two-pass grounding. A coarse pass finds where in the clip the event happens; a fine pass reasons over that window in detail. This mirrors current best practice in accident-scene VLM research rather than asking one pass to do everything.

- Every claim is traceable. If a number appears in the report, it should be possible to point at the exact pipeline stage that produced it.

- Uncertainty is a first-class output. Low-confidence claims are labeled as such, not smoothed over into confident-sounding prose.

# 7. Investigation Report — Output Schema

- Executive Summary — one paragraph, plain language

- Event Timeline — timestamped sequence of what happened

- Vehicle Kinematics Table — per-vehicle speed/acceleration over time (including an ego_vehicle entry), with confidence interval and provenance (fused vs. vision-only)

- Contributing Factors — what the footage supports (e.g. following distance, signal state) — described, not adjudicated

- Evidence Frames — key frames with bounding boxes / speed overlays burned in

- Confidence Appendix — every claim mapped to High/Medium/Low confidence and its source module

- Driver State Observations — eyes-on/off-road, hands-on-wheel, head orientation from the driver-facing camera, kept in its own report section and reported as observation only, never as cause

- Disclaimer — AI-assisted draft, not a certified forensic or legal document

# 8. Phased Delivery Plan

Ten phases, each independently shippable and testable — you can demo after any phase, not just at the end.

## Phase 1 — Foundations & Ingestion Pipeline

| Objective | Stand up the repo, FastAPI skeleton, and the intake path for accident clips. |
| --- | --- |
| Key Deliverables | - POST /ingest endpoint: upload + validate (codec, resolution, duration, corruption check) - Supabase/Postgres schema: Incident, VideoAsset, ReportJob tables - LangGraph StateGraph scaffold (compiled, not a manual for-loop, from day one) - Basic async job queue so ingestion doesn't block the API |
| Primary Tech | FastAPI, Supabase, LangGraph |
| Exit Criteria | A clip can be uploaded, metadata persisted, and a job appears with status "queued". |

## Phase 2 — Video Preprocessing & Super-Resolution

| Objective | Normalize incoming footage and conditionally upscale/stabilize choppy, low-resolution clips before anything else touches them. |
| --- | --- |
| Key Deliverables | - Quality-assessment module scoring blur, noise, resolution, and bitrate - Conditional gate: clips below a quality threshold are routed through FlashVSR (fallback: Real-ESRGAN on constrained hardware) - Frame-rate normalization and basic deinterlacing/stabilization |
| Primary Tech | FlashVSR, Real-ESRGAN, OpenCV |
| Exit Criteria | A visibly choppy 480p test clip is upscaled and stabilized with a measurable SSIM/PSNR improvement over the raw input. |

## Phase 3 — Perception Layer — Detection & Tracking

| Objective | Detect and track every vehicle, pedestrian, and relevant road object frame-by-frame. |
| --- | --- |
| Key Deliverables | - YOLOv11 integration tuned for vehicles/pedestrians/two-wheelers - ByteTrack (or BoT-SORT) multi-object tracking across occlusion - Per-object trajectory log: bbox, class, track ID, frame index, timestamp |
| Primary Tech | YOLOv11, ByteTrack |
| Exit Criteria | Stable track IDs are maintained for every involved road user across the full clip, including brief occlusions. |

## Phase 4 — Ego-Motion & Kinematics Fusion Engine

| Objective | Convert per-vehicle trajectories into real-world speed and acceleration — grounded first in measured ego-motion from the vehicle's own OBD/GPS/IMU, with vision solving only the easier relative-motion problem. Fisheye undistortion runs before any of this and is non-negotiable: a homography fitted to a warped ground plane produces speeds that look plausible but are wrong. |
| --- | --- |
| Key Deliverables | - Fisheye undistortion (per-device intrinsics + distortion coefficients, checkerboard-calibrated once per camera module) — runs first, before every downstream vision step - Per-device CameraProfile/DeviceProfile registry in Supabase: intrinsics, fisheye coefficients, mount height, pitch, yaw - ego.py — ego speed/heading series from OBD + GPS + IMU; the primary kinematic input, not a fallback - relative.py — vision-only relative motion of tracked vehicles vs. the ego frame, scale-anchored by lane-width prior (~3.0-3.5m) or vanishing-point + mount-height geometry, with Depth Anything V2 as a last-resort relative-depth source - absolute.py — absolute_speed(other_vehicle) = ego_speed(OBD/GPS) ⊕ relative_speed(vision), with propagated error bounds - RAFT optical-flow cross-check as an independent second vision estimate; divergence beyond tolerance is flagged, not averaged in - Vision-only fallback when no sensor log exists: every absolute speed is labeled method="vision-only", confidence="low" |
| Primary Tech | OpenCV (fisheye undistortion), custom ego-motion parser (OBD/GPS/IMU), Depth Anything V2, RAFT, custom homography module |
| Exit Criteria | Speed estimates on a labeled test clip (known ground-truth speed) fall within a stated, documented error margin, and every absolute speed in the output carries a provenance tag identifying fused ego+vision vs. vision-only. |

## Phase 5 — Event Detection, Sensor-Log Alignment & Driver State

| Objective | Use the impact sensor / IMU G-force spike as the hard, exact trigger for event-window selection — motion-energy detection doesn't work on a moving camera — align the sensor-log and video timelines, and capture observable driver state at the moment of the incident from the second, driver-facing camera. |
| --- | --- |
| Key Deliverables | - Event window selection from the sensor log: impact_timestamp ± 10s, taken directly from the impact sensor / IMU spike / airbag optocoupler tap - Fallback event detection when no sensor log is present: spike in mean optical-flow-magnitude residual (sudden deceleration shows up as a global flow discontinuity), flagged lower confidence - Timestamp alignment service between video timeline and sensor-log timeline - Discrepancy flagging when vision-estimated speed and logged OBD/IMU speed diverge beyond tolerance - Driver-facing camera pass: eyes-on/off-road, hands-on-wheel, head orientation, reported as observation only, never as cause — kept in a separate driver_state_observations report section |
| Primary Tech | Custom event-detection/fusion service, Pandas/NumPy, OpenCV (driver-facing pass) |
| Exit Criteria | Event window is correctly identified from the sensor-log impact timestamp on clips that have a log, and from the optical-flow-residual fallback on clips that don't; driver-facing observations appear in their own report section with no causal language. |

## Phase 6 — Video-LLM Reasoning Core

| Objective | Feed the processed video plus structured perception data (tracks, speeds, timestamps) into the video-LLM to produce a grounded, causal account of the event. |
| --- | --- |
| Key Deliverables | - Two-pass grounding prompt design: coarse pass to localize the event window, fine pass for detailed step-by-step reasoning - Structured JSON output contract: who / what / when / where / how, each claim timestamp-cited - Injection of perception-layer data (tracks, speed table) into the prompt so the model reasons over real numbers, not just pixels |
| Primary Tech | Qwen3-VL (self-hosted) or Gemini 2.5/3 Pro |
| Exit Criteria | The model returns a structured, timestamp-grounded account of the event that is consistent with the detected tracks and speed data — not a free-form paragraph. |

## Phase 7 — Investigation Report Generation

| Objective | Turn the structured LLM + kinematics output into a professional-looking investigation report. |
| --- | --- |
| Key Deliverables | - Report schema: Executive Summary, Event Timeline, Vehicle Kinematics Table, Contributing Factors, Evidence Frames, Confidence Appendix - Jinja2 templates → WeasyPrint PDF rendering - Annotated evidence-frame exporter (bounding boxes + speed overlays burned into key frames) |
| Primary Tech | Jinja2, WeasyPrint |
| Exit Criteria | A clip goes in and a downloadable, formatted PDF investigation report comes out end-to-end. |

## Phase 8 — Guardrails, Confidence Scoring & Hallucination Mitigation

| Objective | Make sure every claim in the report is grounded in something the pipeline actually detected, and clearly flag anything that isn't. |
| --- | --- |
| Key Deliverables | - Claim-verification pass: cross-check every LLM narrative statement against detection/tracking/kinematics data - Confidence bands (High / Medium / Low) attached per claim - Explicit uncertainty language for anything that can't be grounded, instead of a confident guess - Regression test suite of known clips with expected claims |
| Primary Tech | Custom verifier service, pytest |
| Exit Criteria | Every numeric claim (speed, distance, time) is traceable to a source module, and the regression suite shows an acceptably low rate of ungrounded claims. |

## Phase 9 — API, Dashboard & Integration

| Objective | Expose the full pipeline through a clean API and a usable frontend, and optionally bridge it into VigilNetra's existing hardware-side report module. |
| --- | --- |
| Key Deliverables | - POST /investigate: video in, report out; job-status polling or WebSocket for progress - React + Vite dashboard: upload, live progress, report viewer, evidence-frame gallery - Optional bridge into the existing app/reports module in the VigilNetra hardware repo |
| Primary Tech | FastAPI, React/Vite, WebSockets |
| Exit Criteria | A user can upload a clip through the dashboard and view/download a formatted report without ever touching the API directly. |

## Phase 10 — Testing, Benchmarking, Deployment & Packaging

| Objective | Validate accuracy against real benchmarks, harden the pipeline, and package it so it's demo- and interview-ready. |
| --- | --- |
| Key Deliverables | - Benchmark run against public crash-understanding datasets (e.g. CrashSight-style clips) plus your own collected footage - Load testing on concurrent uploads - Docker Compose packaging with a one-command deployment runbook - A short demo script for showcasing the system |
| Primary Tech | Docker Compose, pytest, locust (load testing) |
| Exit Criteria | Documented accuracy/latency benchmarks exist, deployment is reproducible in one command, and the system is demo-ready. |

# 9. Risks & Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| LLM hallucinates details not in the footage | Report states something false or unverifiable | Mandatory claim-verification pass (Phase 8); uncertainty labeling instead of confident guessing |
| Monocular speed estimation error without known camera calibration | Speed numbers in the report are inaccurate | Ego speed is measured directly from OBD/GPS rather than estimated, so only the relative-motion term is vision-derived; always attach an explicit error margin and cross-check with optical flow. |
| "Investigation report" framing implies legal weight | Report could be over-trusted or misused | Explicit disclaimer baked into every report; language kept descriptive, not adjudicative |
| Large video-LLM inference cost/latency | Slow or expensive to run per clip | Local quantized 32B variant for dev/demo; burst to cloud GPU only when needed |
| Poor generalization to novel scenes (night, rain, two-wheelers) | Lower accuracy on real-world Bengaluru traffic conditions | Benchmark suite explicitly includes low-light and two-wheeler clips, not just clean daytime footage |

# 10. Success Metrics

- Speed estimation within a documented error margin on calibrated test clips (target defined once Phase 4 baseline is measured)

- Claim-groundedness rate from the Phase 8 regression suite above an agreed threshold

- End-to-end latency from upload to PDF report within a usable demo window

- Measurable SSIM/PSNR uplift from the super-resolution stage on degraded test clips

# 11. Open Questions

- Target hardware for inference — local RTX 6600 XT-class desktop only, or budget for cloud GPU bursts?

- How much labeled ground-truth footage (known real speed) can realistically be collected for Phase 4 calibration?

- Given VigilNetra hardware always logs OBD/GPS/IMU, is a vision-only bare-clip mode (no sensor log) worth fully supporting for v1, or should it be flagged unsupported/low-confidence and deprioritized?

- Self-hosted Qwen3-VL vs. Gemini API as the default — a call best made once Phase 6 gives real accuracy/latency numbers on both.

# 12. Appendix — Glossary

VLM: Vision-Language Model — a model that takes images/video and text together

VSR: Video Super-Resolution — upscaling and cleaning up low-quality video

Temporal grounding: A model's ability to point to the specific moment in a video where something happens

Homography: A geometric mapping between two planes — used here to convert pixel positions to real-world ground positions

StateGraph: LangGraph's compiled graph representation of a multi-step pipeline